# Short Comparison: Python 3 vs. ROS 2

For the RockyBorg project using a Raspberry Pi 4 (8 GB RAM), two programming approaches were considered: Classical Python 3 and ROS 2.

| Criteria | Python 3 | ROS 2 |
| --- | --- | --- |
| Learning difficulty | Easier to learn for software beginners | More difficult (require certain amount of experience) |
| Installation | Simple (runs directly on windows 11 with VSCode) | More complicated (ROS is not natively developed for windows, requiring virtual Linux machines for best developing environment.) |
| Programming | Basic Python | Python + ROS 2 concepts |
| Development time | Shorter | Longer |
| Camera and sensor control | Direct Python libraries | ROS 2 nodes |
| Debugging | Easier | More complicated |
| CPU and RAM usage | Generally lower overhead | Additional overhead |
| Advanced navigation and SLAM | More manual integration | Better existing tools |
| Short-term project | Recommended | Less suitable |

# Recommendation (for now, subject-to-change): Classical Python 3

Python 3 is the better choice for this project, mainly because the development team consists of three members with limited programming experience and the project has a limited development period.

Python 3 offers several advantages:

Easier to learn: Fewer programming concepts compared with ROS 2.

Faster development: Existing libraries such as OpenCV, Picamera2, and sensor drivers can be used directly.

Simpler debugging: Problems are easier to identify and fix.

Teamwork: Each member can work on a separate part, such as camera processing, obstacle detection, or IoT data collection.

Sufficient functionality: Python can handle black-line following, basic obstacle avoidance, sensor readings, and MQTT communication with Node-RED.

# Conclusion

Classical Python 3 is recommended for the RockyBorg project. It is simpler, requires less preparation, and is more suitable for a beginner team working under time constraints.

ROS 2 provides useful tools for advanced robotics, but its additional complexity is not necessary for the current project. It can be considered for future development involving advanced SLAM and more advanced autonomous navigation.

For the current project, classical Python offers the best balance between simplicity, performance, and development effort.